Municipal Inventory Information

| LOCATION |  |
| --- | --- |
| LGA Reference Number | 78/41 |
| Name of Place | Concrete Bunkers |
| Location/Address |  |
| Street Name | Great Eastern Highway |
| Suburb/Town | Merredin Townsite |
| Other Locational descriptor | Rear of BP Service Station, between Cohn and Allbeury Streets |
| Local Government Authority | Merredin |
| Map References |  |
| Map Name | Merredin Townsite |
| Map Scale | 1:2,000 |
| Northing | 6515900 m N |
| Easting | 620400 m E |

| OWNERSHIP & LAND DESCRIPTION |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Owner |  | Address |  |  | Phone/fax |  | Status |  | Item No. |  |  |
| Land Description |  |  |  |  |  |  |  |  |  |  |
| Reserve No. | Lot/Location No. |  |  | Plan/Diagram |  | Vol/Folio |  |  | Item No. |  |

| DESCRIPTION |  |
| --- | --- |
| Construction Date (1) | c 1941 |
| Site Type | Defence Installation |
| Use(s) of Place: |  |
| Original | Bunker |
| Present | Nil |
| Construction Materials |  |
| Walls | Concrete |
| Roof | Concrete |
| Other | Windows steel frame; some brick and stone used in steps infill |
| Modifications | Upper floor structure removed in one instance |
| Condition | Fair |
| Integrity |  |
| The fabric of the buildings remains intact providing a high level of integrity |  |  |
| Description |  |
| These two structures of WWII vintage are about 50 m apart, with different orientation. Both are sunk into the ground, which is naturally flat. One protrudes about a metre, has a flat concrete roof and highlight window openings close to ground level at one end; the other end is more open but is now filled with earth. This building has steps to the roof, which is now devoid of any structure. The other structure is sunk lower into the ground and has earth forming a slight mound over the top. At each end there are sloping rectangular openings approximately 2 m x 4 m which appear to be hatch openings with doors removed, suggesting this building was used for storage. Immediately behind one of the hatch openings is a wall about 1 m high, with tapered ends, probably indicating the extent of the structure below ground. |  |  |
| Those parts of the structures that remain, being made of concrete, are likely to suffer little further deterioration. |  |  |

| SIGNIFICANCE |  |  |
| --- | --- | --- |
| Historic theme (s) | Outside Influences - World War II |  |
| Statement of Significance |  |  |
| Local knowledge states that these buildings were constructed as part of a base for counter-attack on Perth and Fremantle, should the metropolitan area have been taken by the Japanese during World War II. This conflicts with the commonly held view that if the enemy had landed in WA, Australia’s defence forces would abandon the State and pull back to a line between Adelaide and Brisbane. These structures are considered to be of national importance. |  |  |
| Recommendation/ Conservation Strategy | Recognise and encourage protection through the Town Planning Scheme. Photographically document prior to any modifications or disturbance. Undertake further research into original use. No immediate conservation measures required. These structures should be considered for inclusion in the State Register of Heritage Places and Register of the National Estate. |  |

| OTHER |  |
| --- | --- |
| Listing and Assessment: |  |
| Assessor (s) Name | Assessor (s) Address/Phone |
| Max Hipkins | WHELANS, 133 Scarborough Beach Rd, Mt Hawthorn, WA, 6016 Tel: (09) 443 1511 |
| Film/Neg. Nos, date | 6/2-4 18.12.95 |
| State Register of Heritage Places | No |
| Classified by the National Trust | No |
| Register of the National Estate | No |
| Local Town Planning Scheme | No |